FLOORPANEL

Technical Data Sheet

| Wood type: | Spruce/fir Others on request |  |  |
| --- | --- | --- | --- |
| Quality: | N: Normal; for visible applications I: Industrial; for constructions with lower aesthetic requirements |  |  |
| Strength class: | C24 |  |  |
| Panel thickness: | 100 to 260 mm Others on request |  |  |
| Panel width: | 150 to 1000 mm All widths within this range are possible. Billed per 50 mm increments. Others on request |  |  |
| Length: | According to list |  |  |
| Laminate widths: | Indefinite, 40 to 70 mm |  |  |
| Surface: | All sides planed and chamfered; for visible applications, sanding the surface is recommended (not standard) |  |  |
| Profile: | Tongue–tongue profile is standard Others on request |  |  |
| Specific weight: | 5.0 kN/m3 (according to SIA 261) |  |  |
| Moisture content: | 12% ± 2% |  |  |
| C24 Mechanical Properties: | Bending strength | fmd | = 14.0 N/mm2 |
| SIA 265, w= 1.0, t= 1.0 | Shear strength | fvd | = 1.5 N/mm2 |
|  | Modulus of Elasticity | Em,mean | = 11.0 kN/mm2 |
| Physical properties: | Thermal conductivity |  | = 0.13 W/mK |
|  | Specific heat | c | = 2.0 - 2.4 kJ/kgK |
|  | Water vapour permeability |  | = 0.035 - 0.015 mg/mhPa |
|  | Diffusion Resistance Factor |  | = 20 - 40 |
| Reaction to fire: | 4.3 (D-s2, d0 according to European Standards) |  |  |
| Gluing: | Polyurethane (PUR) Type I adhesive per EN 15425 Solvent-free (no formaldehyde) Colourless glue lines |  |  |